# Department of Computer Science and Engineering
Islamic University of Technology (IUT)
A subsidiary organ of OIC

# Lab Report 04

# CSE 4412: Data Communication and Networking Lab

## Name: Tahsin Islam
Student ID: 210042137
Section: SWE
Semester: 4th (Summer)
Academic Year: 2022-2023

Date of Submission: 14/02/2024

### Title: Understanding the basics of Inter-VLAN communication using Router, L3 Switch along with basics of Static Routing

### Objectives:

- Implement Inter-VLAN Routing via Router-on-a-Stick.

- Implement Inter-VLAN Communication with Layer 3 Switch.

- Demonstrate Static Routing.

- Ensure Proper Segmentation and Security.

- Document and Provide Clear Explanations.

### Task 1:

Step 1: Configuring Switches

At first, I configured 3 VLANs with VLAN IDs 10, 20, and 30 inside the switch and assigned appropriate names. To do this, I have used the following commands:

Switch> enable Switch# config t Switch(config)# vlan 10 Switch(config-vlan)# name student Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# name teacher Switch(config-vlan)# exit Switch(config)# vlan 30 Switch(config-vlan)# name admin Switch(config-vlan)# exit

Then I configured the Interfaces belonging to each VLAN. For that, I used the following commands:

Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/4 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 30 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit

This way I have configured the interfaces. For Switch2, the commands are exactly same as Switch1. There is a bit of difference in Switch1.

Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/4 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/5 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 30 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit

As Switch5 has 2 trunk connections that’s why the changes in the command.

For Router-on-a-stick, we configured Switch0 with the following:

Switch(config)# interface fastEthernet 0/5 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit

Here are the screenshots of the three switches:

Fig 1.2: Switch0 configuration

Fig 1.3: Switch1 configuration

Fig 1.4: Switch3 configuration

Step 2: Configuring Router

Now for router configuration, I used the following commands on router’s CLI:

Router> enable Router# config t Router(config)# interface gigabitEthernet 0/0 Router(config-if)#no shutdown Router(config)#int g0/0.10 Router(config-subif)#encapsulation dot1q 10 Router(config-subif)#ip address 192.168.47.1 255.255.255.0 Router(config-subif)#exit Router(config)#int g0/0.20 Router(config-subif)#encapsulation dot1q 20 Router(config-subif)#ip address 192.168.57.1 255.255.255.0 Router(config-subif)#exit Router(config)#int g0/0.20 Router(config-subif)#encapsulation dot1q 20 Router(config-subif)#ip address 192.168.67.1 255.255.255.0 Router(config-subif)#exit

Here are the screenshots of the router:

Fig 1.5: Router configuration

Step 3: Setting Up IPs

As my ID’s last 2 digits are 37, IP addresses are:

VLAN 10 host’s IP address:

- PC0: 192.167.47.2

- PC3: 192.167.47.3

- PC6: 192.167.47.4

VLAN 20 host’s IP address:

- PC1: 192.167.57.2

- PC4: 192.167.47.3

- PC7: 192.167.47.4

VLAN 30 host’s IP address:

- PC2: 192.167.67.2

- PC5: 192.167.67.3

- PC8: 192.167.67.4

Here are the screenshots:

Fig 1.6: IP Address of PC0

Fig 1.7: IP Address of PC1

Fig 1.8: IP Address of PC2

Step 4: Using the ping command from the Terminal

To ping, I clicked on PC0, went to Desktop, then Command Prompt. In the command prompt, I wrote:

Ping 192.167.47.3

We saw that 4 packets were sent, 4 packets were received, 0 packets were lost as they are on VLAN 10.

But when I sent a ping to PC6 using this:

Ping 192.167.57.3

We saw that, 4 packets were sent, 0 packets were received, 4 packets were lost as they are on different VLANs.

But when I sent a ping to PC1 using this:

Ping 192.167.57.2

We saw that, 4 packets were sent, 4 packets were received, 0 packets were lost as they are under same switch.

Here is the screenshot of the command prompt of PC0:

Fig 1.9: Command Prompt of PC0

### Task 2:

Step 1: Configuring Switches

At first, I configured 4 VLANs with VLAN IDs 10, 20, 30, and 40 inside the switch and assigned appropriate names. To do this, I have used the following commands:

Switch> enable Switch# config t Switch(config)# vlan 10 Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# exit Switch(config)# vlan 30 Switch(config-vlan)# exit Switch(config)# vlan 40 Switch(config-vlan)# exit

Then I configured the Interfaces belonging to each VLAN. For that, I used the following commands:

Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/4 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/5 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 30 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/6 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 40 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit

This way I have configured the interfaces. For Switch1, the commands are exactly same as Switch0. The screen shots are given below:

Step 2: Configuring Multilayer Switch

Now for router configuration, I used the following commands on router’s CLI:

Switch> enable Switch# config t Switch(config)# vlan 10 Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# exit Switch(config)# vlan 30 Switch(config-vlan)# exit Switch(config)# vlan 40 Switch(config-vlan)#ip routing Switch(config)#int range fa0/1-2 Switch(config-if-range)#switchport trunk encapsulation dot1q Switch(config-if-range)#switchport mode trunk Switch(config-if-range)#no shutdown Switch(config-if-range)#int vlan 10 Switch(config-if)#ip address 192.168.48.1 255.255.255.0 Switch(config-if)#no shut Switch(config-if)#exit Switch(config-if-range)#int vlan 20 Switch(config-if)#ip address 192.168.49.1 255.255.255.0 Switch(config-if)#no shut Switch(config-if)#exit Switch(config-if-range)#int vlan 30 Switch(config-if)#ip address 192.168.50.1 255.255.255.0 Switch(config-if)#no shut Switch(config-if)#exit Switch(config-if-range)#int vlan 40 Switch(config-if)#ip address 192.168.51.1 255.255.255.0 Switch(config-if)#no shut Switch(config-if)#exit

Here are the screenshots of the switch:

Step 3: Setting Up IPs

As my ID’s last 2 digits are 37, IP addresses are:

VLAN 10 host’s IP address:

- PC0: 192.167.48.2

- PC4: 192.167.48.3

VLAN 20 host’s IP address:

- PC2: 192.167.49.2

- PC5: 192.167.49.3

VLAN 30 host’s IP address:

- PC2: 192.167.50.2

- PC5: 192.167.50.3

VLAN 40 host’s IP address:

- PC2: 192.167.51.2

- PC5: 192.167.51.3

Step 4: Using the ping command from the Terminal

To ping, I clicked on PC0, went to Desktop, then Command Prompt. In the command prompt, I wrote:

Ping 192.167.48.3

We saw that 4 packets were sent, 4 packets were received, 0 packets were lost as they are on VLAN 10.

But when I sent a ping to PC6 using this:

Ping 192.167.49.3

We saw that, 4 packets were sent, 0 packets were received, 4 packets were lost as they are on different VLANs.

But when I sent a ping to PC1 using this:

Ping 192.167.49.2

We saw that, 4 packets were sent, 4 packets were received, 0 packets were lost as they are under same switch.

### Task 3:

### Questions (Answer to the point):

      - Why do we need L3 Switches?

Ans: L3 switches are needed to perform routing functions at the network layer (Layer 3) of the OSI model.

      - What is the use router in Inter-Vlan Routing?

Ans: Routers are used in Inter-VLAN Routing to facilitate communication between different VLANs. They route traffic between VLANs, allowing devices in different VLANs to communicate with each other.

      - What changes are needed while configuring VLANs using L3 switches instead of Router-on-a-stick approach?

Ans: When configuring VLANs using L3 switches instead of the Router-on-a-stick approach, changes involve configuring VLAN interfaces directly on the switch and enabling routing between them. This eliminates the need for a separate router interface for each VLAN and allows for faster inter-VLAN communication within the switch itself.

      - What is next-hop floating address?

Ans: A next-hop floating address is a backup route used in routing protocols such as OSPF or EIGRP. It is configured with a higher administrative distance than the primary route and is used only if the primary route becomes unavailable.

      - What is the disadvantage of static routing?

Ans: A disadvantage of static routing is its lack of flexibility and scalability. Static routes require manual configuration and do not adapt to changes in the network topology automatically. This makes them less suitable for large or dynamic networks where routing needs may change frequently.

### Challenges (if any):

No challenges faced