الجامعة الإسلامية للتكنولوجيا

UNIVERSITÉ ISLAMIQUE DE TECHNOLOGIE

ISLAMIC UNIVERSITY OF TECHNOLOGY

DHAKA, BANGLADESH

ORGANIZATION OF ISLAMIC COOPERATION

CSE 4412

Data Communication and Networking Lab

Lab-03 Report

Name			: Tahsin Islam

Student ID		: 210042137

Department		: Computer Science and Engineering

Program			: B.Sc. in Software Engineering

Semester			: Summer 2022-2023 ( 4th )

Submission Date	: 07/02/2024

Task-01

Fig 1.1: Topology

Step 1: Set the IP Address

As I can connect a maximum of 37 (last 2 digits of my ID) hosts per network, in the last octet of the Subnet we will need 5 bits for the host address (26=64), and the first 2 bits of the last octet will be part of the network address.

Subnet: 255.255.255.192

IP range of the first subnetwork: 192.168.47.1 – 192.168.47.64

Default gateway of subnetwork 1st subnetwork: 192.168.47.1

IP range of the second subnetwork: 192.168.57.1 – 192.168.57.64

Default gateway of subnetwork 2nd subnetwork: 192.168.57.1

The screenshots of two PCs of each network are given below:

Fig 1.2: IP Address of PC0

Fig 1.3: IP Address of PC3

Step 2: Router Configuration

To configure the router, I clicked on the router and went to CLI. Ther used the following commands:

Router# enable Router# config t Router(config)# interface gigabitEthernet 0/0 Router(config-if)# ip address 196.168.47.1 255.255.255.192 Router(config-if)# no shutdown Router(config-if)# exit Router(config)# interface gigabitEthernet 0/1 Router(config-if)# ip address 196.168.57.1 255.255.255.192 Router(config-if)# no shutdown Router(config-if)# exit

Here is the screenshot of the commands:

Fig 1.4: Router configuration

Step 3: Using the ping command from the Terminal

To ping, I clicked on PC0, went to Desktop, then Command Prompt. In the command prompt, I wrote:

Ping 192.168.57.10

Here is the screenshot of this step:

Fig 1.5: Command Prompt of PC0

We can see that 4 packets were sent, 4 packets were received, and 0 packets were lost.

Task-02

Fig 2.1: Topology

Step 1: Configuring Switches

At first, I configured 3 VLANs with VLAN IDs 10, 20, and 30 inside the switch and assigned appropriate names. To do this, I have used the following commands:

Switch> enable Switch# config t Switch(config)# vlan 10 Switch(config-vlan)# name student Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# name teacher Switch(config-vlan)# exit Switch(config)# vlan 30 Switch(config-vlan)# name admin Switch(config-vlan)# exit

Then I configured the Interfaces belonging to each VLAN. For that, I used the following commands:

| Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 Switch(config-if)# no shutdown Switch(config-if)# exit |  |
| --- | --- |
| Switch(config)# interface fastEthernet 0/4 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 30 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit |

This way I have configured the interfaces. For Switch6, the commands are exactly same as Switch9. There is a bit of difference in Switch5.

Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/4 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/5 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 30 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan all Switch(config-if)# no shutdown Switch(config-if)# exit

As Switch5 has 2 trunk connections that’s why the changes in the command.

Here are the screenshots of the three switches:

Fig 2.2: Switch9 configuration

Fig 2.3: Switch5 configuration

Fig 2.4: Switch6 configuration

Step 2: Setting Up IPs

As my ID’s last 2 digits are 37, IP addresses have started from 192.167.47.1.

But for all the VLAN 10 hosts, I have set their IP address with the last digit 2, for VLAN 20 hosts, last digit is 3, and for VLAN 30 hosts, last digit is 4.

VLAN 10 host’s IP address:

- PC0: 192.167.47.2

- PC3: 192.167.47.12

- PC6: 192.167.47.22

VLAN 20 host’s IP address:

- PC0: 192.167.47.3

- PC3: 192.167.47.13

- PC6: 192.167.47.23

VLAN 30 host’s IP address:

- PC0: 192.167.47.4

- PC3: 192.167.47.14

- PC6: 192.167.47.24

Here are the screenshots:

Fig 2.5: IP Address of PC0

Fig 2.6: IP Address of PC1

Fig 2.6: IP Address of PC2

Step 3: Using the ping command from the Terminal

To ping, I clicked on PC0, went to Desktop, then Command Prompt. In the command prompt, I wrote:

Ping 192.167.47.12

We saw that 4 packets were sent, 4 packets were received, 0 packets were lost as they are on VLAN 10.

But when I sent a ping to PC6 using this:

Ping 192.167.47.24

We saw that, 4 packets were sent, 0 packets were received, 4 packets were lost as they are on different VLANs.

Here is the screenshot of the command prompt of PC0:

Fig 2.7: Command Prompt of PC0

### Questions (Answer to the point):

      - How many host bits are needed in the largest required subnet?

Ans:  6 bits.

      - How many VLANs need to be configured to each of the switches?

Ans: 3 VLANs.

      - Which interfaces need Access Link?

Ans: Interfaces that are connected with PCs.

      - Which interfaces need Trunk Link?

Ans: Interfaces that are connected with another switch.

      - After configuring VLAN, what will happen if we broadcast?

Ans: After configuring VLANs, broadcast traffic will only be forwarded within the same VLAN, limiting its scope.

### Challenges (if any):

No challenge faced.